Sliding 4th Temperament

A4A3D4C#4C#3F3F4C#4C#3A#3F#3B3G3C4G#3D#4E4

Barber Temperament

A4A3F3F4A#3C4D4F#3B3C#4D#4G3E4G#3

Crabb Temperament

A4A3D4G3C4F3F4A#3D#4G#3C#4F#3B3E3

Coleman Temperament

A4D4A3F3A#3F4C#4F4F3A#3F#3E4G3B3C4D#4G#3

Defebaugh Temperament

A4A3F3D4A#3C#4G#3C4F#3D#4B3G3E4F4

Sliding 4th Step by step

A4 – Tune to A440 with Fork				Check with F2 listening at A4

A3 – Tune to A4 listening at A5 (almost pure)		Check with F3 (10th slightly faster)

D4 – Tune to A3 listening at A5 (1bps)	Check with F3 (6th is 1bps faster) 		 Check with A4 listening at A5

C#4 – Tune a fast discernable beat at C#6		No Check

C#3 – Tune to C#4 listening at C#5 (almost pure)		Check with A2

F3 – Set with contiguous 3rds listen at 5th & 6th octave	C#3-F3/F3-A3/A3-C#4

Confirm that F3-A3 is close to 7bps

F4 – Tune to F3 listening at F5 (almost pure)		Check with C#3

C#4 – Set with contiguous 3rds listen at 5th & 6th octave	F3-A3/A3-C#4/C#4-F4

C#3 – Tune to C#4 listening at C#5 (almost pure)		Check with A2

Confirm steady transition of contiguous 3rds from C#3 to A4

A#3 – Tune to F3 listening at F5 (1bps)			Check with C#3 (6th is 1bps faster) 									Check with F4 listening at F5 										Check that A#3-D4 is slightly faster than A3-C#4								listening at the 6th octave

F#3 – Tune to A#3 listening at A#5 (faster than F3-A3)	Check with C#4 listening at C#5

B3 – Tune to F#3 listening at F#5 (1bps)			Check with D3 (6th is 1bps faster)

G3 – Tune to B3 listening at B5 (faster than F#3-A#3)	Check with D4 listening at D5										Check with F3-D4 listening at A5 (same speed)

C4 – Tune to G3 listening at G5 (1bps)			Check with F3 listening at C5										Check with F4 listening at C6

G#3 – Tune to C4 listening at C6 (faster than G3-B3)	Check with C#4 listening at G#5 (1bps)									Check with F4 against A#3-D4 (same speed)

Walk the 3rds, 4ths, & 5ths before moving on and make minor corrections to ensure they are all even.

D#4 – Tune to F#3 listening at A#5 (faster than F3-D4)	Check with G#3 listening at D#5										Check with A#3 listening at A#5										Check with B3 (faster than A#3-D4)

E4 – Tune to G3 listening at B5 (faster than F#3-D#4)	Check with A3 listening at E5										Check with B3 listening at B5										Check with C4 (faster than B3-D#4)

Walk the 6ths, 5ths, 4ths, & 3rds listening for consistent beat rates.